| GENERAL INFORMATION |  |
| --- | --- |
| SIMPLWINDOWS NAME: | Vanco EVSW1040 4x1 HDMI Switch |
| CATEGORY: | Switches |
| VERSION: | 1.0 |
| SUMMARY: | This module integrates Vanco 4X1 HDMI Switch (EVSW1040) via RS-232. |
| GENERAL NOTES: | This module integrates the functions of a Vanco 4X1 HDMI Switch (EVSW1040) via RS-232. |
| CRESTRON HARDWARE REQUIRED: | 3 series processor. |
| SETUP OF CRESTRON HARDWARE: | Connect the Vanco 4X1 HDMI Switch to a RS-232 serial port on the Crestron controller. Configure the COM Two-way serial driver to the connected RS-232 serial port with the following settings: Baud Rate - 115200 Data Bits - 8 Stop Bits - 1 Parity - N Connect "TX_SERIAL_COMMAND" to the [tx$] connection of the Two-way serial driver. Connect "RX_SERIAL_DATA" to the [rx$] connection of the Two-way serial driver. |
| VENDOR FIRMWARE: | V1.0.1 |
| VENDOR SETUP: | N/A |
| CABLE DIAGRAM: | N/A |

| CONTROL: |  |  |
| --- | --- | --- |
| SET_INPUT_D[1-4] | D | Pulse to select an input to switch to. |
| SET_INPUT_A | A | Pulse [1-4] to select an input to switch to. |
| SET_MODE | A | Pulse [0-2] to set the display mode to the pulsed number. 0 is QUAD, 1 is PIP, and 2 is SINGLE. |
| SET_CONTRAST | A | Pulse [0-99] to adjust the contrast to the pulsed number. |
| SET_BRIGHTNESS | A | Pulse [0-99] to adjust the brightness to the pulsed number. |
| SET_COLOR | A | Pulse [0-99] to adjust the color to the pulsed number. |
| SET_SHARPNESS | A | Pulse [0-8] to adjust the sharpness to the pulsed number. |
| SET_RESOLUTION | A | Pulse [0-4] to set the resolution to the pulsed number. 0 is 1080p, 1 is 720p, 2 is 1080i, 3 is 1024x768, and 4 is 1360x768. |
| SET_OSD | A | Pulse 0 to disable OSD and 1 to enable OSD. |
| RX_SERIAL_DATA | S | Connect to the [rx$] output of the Two-way serial driver. |

| FEEDBACK: |  |  |
| --- | --- | --- |
| CURRENT_INPUT | A | Analog feedback of the currently selected input. |
| CURRENT_MODE | A | Analog feedback of the currently selected display mode. |
| CURRENT_CONTRAST | A | Analog feedback of the current contrast. |
| CURRENT_BRIGHTNESS | A | Analog feedback of the current brightness. |
| CURRENT_COLOR | A | Analog feedback of the current color. |
| CURRENT_SHARPNESS | A | Analog feedback of the current sharpness. |
| TX_SERIAL_COMMAND | S | Connect to the [tx$] input of the Two-way serial driver. |

| TESTING: |  |
| --- | --- |
| OPS USED FOR TESTING: | RMC3 1.010.0060 |
| SIMPL WINDOWS USED FOR TESTING: | 4.03.20 |
| DEVICE DB USED FOR TESTING: | 69.05.001.00 |
| CRES DB USED FOR TESTING: | 53.05.005.00 |
| SYMBOL LIBRARY USED FOR TESTING: | 968 |
| SAMPLE PROGRAM: | Vanco EVSW1040 4x1 HDMI Switch |
| REVISION HISTORY: | v1.0 – Original Release |